# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply and Delivery of Electric Vehicle

# p 1 /3

Publication reference: BGTR0200103-PP2-SUP-01

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 Electric Vehicle | Performance and Battery Specifications Fuel Type: Fully electric Power Output ≥ 190 kW Energy Consumption (WLTP combined or equivalent): 20 kWh/100 km (up to 5% higher value acceptable), as evidenced by official manufacturer documentation / type-approval data. Acceleration: 0-100 km/h ≤ 9 seconds (preferred) Drivetrain: Preferably all-wheel drive (AWD) Driving Range: Minimum 340 km (WLTP combined) DC Charging Power ≥ 90 kW Battery Capacity (Usable Energy): Minimum 65 kWh Charging: DC fast charging and AC charging support Dimensions and Capacity Seating Capacity: Minimum 7 seats Length: Minimum 4,500 mm Width: Minimum 1,900 mm Luggage Capacity: Adequate and flexible luggage capacity |  |  |  |
|  | Comfort and Technology Seats: Heated (optional/preferred) and adjustable front seats Climate Control: Dual-zone automatic climate control system Roof: Panoramic glass roof (optional/preferred) Displays: Advanced infotainment screen, driver display, and optional head-up display Connectivity: Wireless Android Auto and/or Apple CarPlay support Safety High-performance LED headlights Airbags: Front, side and curtain airbag system Driver Assistance Systems: Lane keeping assist, autonomous emergency braking or active brake assist, tire pressure monitoring system (optional/preferred), adaptive cruise control, parking assist, 360-degree camera system Crash Tests: Must have received high ratings (minimum 4 stars) from Euro NCAP or equivalent safety tests, or demonstrate compliance with equivalent safety standards through official documentation Other Conditions Warranty and Service: Minimum 2-year warranty and the tenderer must ensure that service and maintenance support for the supplied vehicle is available in Türkiye, either through the manufacturer's authorized service network or through a certified maintenance provider. Origin Requirement: All goods supplied under this contract must originate from an eligible country, in accordance with the applicable rules of origin for the Programme and the relevant financing arrangements. Evidence of origin may be requested prior to delivery and/or at acceptance. Inspection and Compliance: During the procurement process and at the time of delivery, the origin of the goods shall be verified. Goods found non-compliant shall not be accepted and shall be replaced by the supplier at its own cost, without prejudice to the Contracting Authority’s rights under the contract. Delivery Period: Delivery shall be completed within a maximum of 120 working days from the day following the date of contract signature. |  |  |  |